ME1/1 HV setup instruction.

Important!!! The working HV for ME1/1 CSCs  is 3000V.

You have to logon on PCDUBNA3  computer through the Remote Desktop Connection

(use mstsc command for WinXp)

User name: ME_1_1

Password: contact HV expert

1. Start dns server if it is not running.

2. Start PVSS II Console and new window PVSS Console will appear. Write the name of the project CMS_ME11_HV_2  and start it (click Green Light button).Waiting when new window Device Editor & Navigator is appeared. If  not, restart  PVSS Console by clicking on Red Light.  Be patient till all processes are finished and after that  click on Green Light .

3. There are three possibilities of  HV crate control.

- Hardware. It convinients for synchronous HV ON/OFF on all CSCs (in  MTCC case  there are  ME1/1/27–ME1/1/32). You have to choose HVME11P by double click on it . In new window click on “HV Action” button , write new parameters and click “Switch channel on”. It is similar to CAEN front panel control.

- Logical. Here you can HV ON/OFF and check all parameters separetely on each plane  of selected CSC . It is helpfully for working with one or few CSCs. First you need to choose CSC, then plain, after that  write new parameters in Setings raw and finally click on Power button.

- FSM. This mode is helpfully for monitoring all HV parameters during  time of  CAEN is ON. First of all you need click  on START/RESTART button in the bottom of the window then click the right  mouse button  on CMS_CSC_ME11P and “View”, later on in the new window click on the lock and  choose “take”- command. This window  must be open during all working time. Finally you can choose any CSC and its  individual  plane for  checking status and  control.

Important!!! The working HV for ME1/1 CSCs  is 3000V.