CSC TrackFinder shift Manual.

Questions/comments: dan.holmes@cern.ch

Panic:			(a)  CSC shift leader phone

(b) Dan’s phone: 165404 (CERN)

Quick configure (crate is running already, someone else has been using the system):

1/ Command Line:

> ssh slice@emutf

> ./initSPForRealData

2/ GUI ..to come 

Take A Run of Trigger Data: (please do for all runs!)

Before the run starts, you have to start the hardware dumper that dumps trigger data as I passes through the DDU.

..on slice@emutf:

>cd /data/

>cd or mkdir of todays date (dd_mm_yy)

>make or edit the readMe.txt, I want to know how the runNumber below corresponds to a global DAQ run number (if global daq) and what the trigger conditions are?

>~/HardwareDumper.exe –r <<runNumber>> -dboff

Run trigger DQM: (nice to do after each run)

> copy one of the data .bin files to emutf:/data/

> in 5 mins look at:

http://www.phys.ufl.edu/cms/emu/dqm/dqm_results/SliceTest/TrackFinder/browse.html

> send a mail/message to dan.holmes@cern.ch

Switch Trigger Criteria:

..on slice@emutf:

>cd ~/dan

>run one of the following scripts:{

trig_allLCT.sh   	 you will get a trigger for each LCT that arrives at the SP

trig_ME1.sh		 trigger on any LCT that comes from ME1

trig_ME1a.sh         	 trigger on any LCT from ME1a (DT sector 10)

trig_ME1b.sh             	 trigger on any LCT from ME1b (DT sector 11)

trig_ME2.sh          	 trigger on any LCT from ME2

trig_ME3.sh		 trigger on any LCT from ME3

trig_coincidence.sh     trigger on a coincidence of LCTs from any 2 stations (default)

trig_noTrigs.sh   	 turn off triggers

trig_readTrigs.sh	 prints out current trigger mode, match the register setting with on in above scripts to work out which mode we’re in. }

Full configure (TF crate is off):

> switch on crate power

> switch on external +12V supply (located on top of back of TF crate)

> ssh slice@emutf

> ./loadLUTs

> now follow ‘Quick configure’

PC has been rebooted: (you need root pwd)

>ssh slice@emutf

>cd dan

>./loadCAEN_TF

>./loadGBCard

Debug:

1/ LEDs:

CCB01 (slot 12 of TF crate):

All 3 of the green power LEDs at the bottom of the CCB01 should be on:

>If the -5.2V one if off then check the external 12V power supply located on top of the back side of the TF crate.

>If either the +5v or the +3v is off but the TF crate is on then phone the numbers at the top of the page.

‘TTC’ :top, yellow. should be on.  This means the crate is using an external clock source rather than an internal one.  If it is not on then follow the ‘quick configure’ steps.  It should come on as part of the ‘initSPforRealData’.

‘L1A’, red, good, this means the CCB is receiving triggers from the TTC system.

‘BX0’, red, good; the CCB is receiving BC0 from the TTC system.

‘SNER’, red, bad; there has been an error in transmission of commands from the TTC system.  We managed to recover the command.

‘DBER’, red, bad; there was an error in transmission of commands from the TTC system.  We did not manage to recover the command.  If running is not obviously effected then a note should be made in the log.  If running is effected (TTC commands missed) then the crate should be power cycled.

SP (slot 7 of TF crate):

‘No lock’, red : bad! this means there is no TTC clock lock, the SP cannot run.

Check the CCB01 ‘TTC’ LED, follow steps above if  off.

Check the TTC system, is it on?  Ask a TTC responsible or the CSC shift leader.

‘L1ACC’, yellow; good.  The TF crate is receiving triggers from the TTC system

‘TRACK’, yellow; good.  The SP is making triggers that should be being sent to the CCB

nb if ‘track’ is on but ‘L1ACC’ is off then the SP is making trigger requests but it is not receiving back the corresponding L1A back from the TTC system.  Debug the trigger path from SP to the TTC and back.  There is a useful scalar in the NIM crate (on end of ribbon cable from CCB01).

‘RDY’, green, good.  The SP is configured.  If this is not on then follow the ‘quick configure’ steps.

‘wof’,red,bad; the DDU readout has not been able to keep up with the trigger rate.  SP FMM will kick in.  Check the TF DDU is alive.

small yellow front panel LEDs, indicate over which links data is being sent, they flash when LCTs are incoming

If ‘track’ is on but ‘L1ACC’ is off then the

2/ s/w Errors when talking to crate:

> check we have all the modules loaded (after last reboot):

>… as slice@emutf

> cd dan

>/sbin/lsmod

..you should see following:

>a2818  ..if not then run ./loadCAENTF

>v2718  ..if not then run ./loadCAENTTC

>dl2k & eth_hook_2  if not then run ./loadGBCard

>”problems with initialising CAEN library CV2718 …status =-2”

>is the crate on!?

>is the controller connected? (directly above the fiber input to the controller there are 2 LEDs.  The one on the right is green and should be on.  If it is not then there is no link to the computer seen here.

>try re-loading the CAEN driver:

>ssh slice@emutf

>./loadCAENTF  ..you will need the root password.

>”problems with initialising CAEN library CVA2818 ….”

>reload CAEN TF driver as above

>”cannot read/write FPGA..  ..check address table”

> if you get this while trying to do ‘initSPForRealData” or one of the “trigModes” functions then call my phone.